Animatic eye

AIM:

To design a vision based animatic eye for mimicking the eye movement of human when it detects a human face.

COMPONENTS:

| SI.NO | COMPONENTS | QUANTITY | PRICE | COMPANY/SITE | ID |
| --- | --- | --- | --- | --- | --- |
| 1 | Servo motor (SG90) | 06 | ₹ 94.00 | ROBU.IN | 5764 |
| 2 | Arduino uno | 01 | ₹ 439.56 | ROBU.IN | 6337 |
| 3 | Sensor Shield v5 | 01 | ₹ 83.00 | ROBU.IN | 26598 |
| 4 | Camera | 01 | ₹ 872.00 | ROBU.IN | 45666 |
| 5 | Battery | 01 | ₹ 891.00 | ROBU.IN | 1237659 |

PROCEDURE:

- Mount the SG90 servos securely inside the animatronic eye mechanism.

- Connect the servos to the microcontroller

- Set servos to 90° (center position) using a simple test code, ensuring the animatic eye face toward in its neutral resting position.

- Connect and configure the camera and run Python/OpenCV code to detect the user’s face and extract its (x, y) location in the camera frame.

- Upload/run the tracking algorithm, which converts the face position into smooth servo movement, allowing the animatronic eye to follow the target realistically.

CIRCUIT DIAGRAM:

ALGORITHMS:

Start

- Initialize the eye servos and set them to the neutral 90° position.

- Start the camera stream and activate the face detection model (MediaPipe).

- Read a frame from the camera and find the x, y position of the detected target in the video feed.

- Compare the target’s position with the center of the camera view and rortate the servos to that position.

- Convert the detected target movement into smooth servo motion

- Apply movement limits to keep motion natural

- Send the updated angles to servos.

- Repeat Steps 3–7 continuously to maintain real-time tracking.

CODE:

Python:

import cv2

import serial

import time

import mediapipe as mp

# =============================================

# SERIAL COMMUNICATION

# =============================================

arduino = serial.Serial('COM5', 115200)   # Change COM port

time.sleep(2)

# =============================================

# MEDIAPIPE FACE MESH INITIALIZATION

# =============================================

mp_face_mesh = mp.solutions.face_mesh

face_mesh = mp_face_mesh.FaceMesh(refine_landmarks=True)

# =============================================

# HELPER: Calculate Eye Aspect Ratio (Blink Detection)

# =============================================

import math

def distance(a, b):

return math.dist([a.x, a.y], [b.x, b.y])

def eye_aspect_ratio(landmarks, idx):

# Eye landmarks indices for blink detection

top = landmarks[idx[1]]

bottom = landmarks[idx[5]]

left = landmarks[idx[0]]

right = landmarks[idx[3]]

vertical = distance(top, bottom)

horizontal = distance(left, right)

return vertical / horizontal

# Mediapipe landmark indices for eye

RIGHT_EYE = [33, 159, 145, 133, 153, 144]

LEFT_EYE  = [362, 386, 374, 263, 380, 373]

# =============================================

# CAMERA LOOP

# =============================================

cap = cv2.VideoCapture(0)

while True:

ret, frame = cap.read()

if not ret:

break

h, w = frame.shape[:2]

# Convert to RGB for mediapipe

rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = face_mesh.process(rgb)

if results.multi_face_landmarks:

mesh = results.multi_face_landmarks[0].landmark

# =============================================

# FACE POSITION FOR EYEBALL MOVEMENT

# =============================================

nose = mesh[1]

x = nose.x * w

y = nose.y * h

# Determine direction

if x < w * 0.40:

eye_dir = "LEFT"

elif x > w * 0.60:

eye_dir = "RIGHT"

else:

eye_dir = "CENTER"

if y < h * 0.40:

eye_vert = "UP"

elif y > h * 0.60:

eye_vert = "DOWN"

else:

eye_vert = "CENTER"

# =============================================

# BLINK DETECTION

# =============================================

ear_left = eye_aspect_ratio(mesh, LEFT_EYE)

ear_right = eye_aspect_ratio(mesh, RIGHT_EYE)

EAR = (ear_left + ear_right) / 2

blink = EAR < 0.20   # threshold

# =============================================

# SEND COMMANDS TO ARDUINO

# =============================================

command = f"{eye_dir},{eye_vert},{int(blink)}\n"

print("Sending:", command)

arduino.write(command.encode())

cv2.imshow("Animatronic Eye Control", frame)

if cv2.waitKey(1) == 27:

break

cap.release()

cv2.destroyAllWindows()

Arduino:

#include <Servo.h>

Servo rightUp;

Servo rightDown;

Servo leftUp;

Servo leftDown;

Servo eyeballH;

Servo eyeballV;

void setup() {

Serial.begin(115200);

rightUp.attach(3);

rightDown.attach(5);

leftUp.attach(6);

leftDown.attach(9);

eyeballH.attach(10);

eyeballV.attach(11);

centerEye();

}

void centerEye() {

eyeballH.write(90);

eyeballV.write(90);

rightUp.write(90);

rightDown.write(90);

leftUp.write(90);

leftDown.write(90);

}

void loop() {

if (Serial.available()) {

String data = Serial.readStringUntil('\n');

data.trim();

int c1 = data.indexOf(',');

int c2 = data.indexOf(',', c1 + 1);

String horizontal = data.substring(0, c1);

String vertical = data.substring(c1 + 1, c2);

int blink = data.substring(c2 + 1).toInt();

// ======================

// EYEBALL MOVEMENT

// ======================

if (horizontal == "LEFT") eyeballH.write(120);

else if (horizontal == "RIGHT") eyeballH.write(60);

else eyeballH.write(90);

if (vertical == "UP") eyeballV.write(120);

else if (vertical == "DOWN") eyeballV.write(60);

else eyeballV.write(90);

// ======================

// BLINK / EYELIDS

// ======================

if (blink == 1) {

// Closed eyes

rightUp.write(90);

rightDown.write(90);

leftUp.write(90);

leftDown.write(90);

} else {

// Open eyes

rightUp.write(140);

rightDown.write(50);

leftUp.write(40);

leftDown.write(130);

}

}

}